# Ovation Golf

Demonstration System Specifications

# Table of Contents

Table of Contents	2

1	Introduction	3

1.1	Background	3

2	Definitions	3

3	System Requirements	4

3.1	Objectives	4

3.2	System Context	5

3.2.1	Use Cases	6

4	System Architecture	6

4.1	LabView Demo	7

4.1.1	Windows Tablet/Laptop	7

4.1.2	LabView Application User Interface	8

4.2	GPS System	9

4.3	Wi-Fi Router	9

5	Test Modes	10

5.1	Connectivity Test	10

5.2	Rover Status	11

5.3	Base Station Status	12

6	Demo Process	12

6.1	Setup	12

6.2	Generating Putt Instructions	13

7	Golf Course Information	14

7.1	Riverside Country Club	14

7.2	Shorehaven Golf Club	14

8	Coordinate Transformations	15

8.1	Coordinate systems	16

8.1.1	Additional Reference Information	17

8.2	Tools	18

# Introduction

## Background

Ovation Golf has developed a unique, innovative system that is able to significantly enhance the putting experience for golfers at all levels of the game. This includes beginners to experienced professionals with a desire to learn how to read a green and execute a successful putt.

The core of the Ovation Golf system is an advanced 3D simulation system that has the following capabilities:

- Enhanced 3D map of the green.

- Able to identify the location of the ball and hole.

- Evaluate hundreds of ball trajectories to determine the ideal putt.

- Communicate to the golfer how to execute a successful putt.

- Educate the golfer on how to read the green.

The Ovation Golf Demonstration System will showcase the functionality of the advanced simulation system by utilizing a COTS GPS system and enhancing existing LabView development software.

# Definitions

The following terms are used in the document.

- Latitude: Latitude in WGS84 format.

- Longitude: Longitude in WGS84 format.

- Altitude: Position above the WGS84 ellipsoid (meters).

- Fix Type

  - Fixed: Position fix that indicates the Base Station has achieve sufficient accuracy for RTK operation.

  - Surveyed: Indicates base station has achieved sufficient accuracy for RTK operation.

  - 3D: Position fix that includes horizontal coordinates (XY) plus altitude.

  - 2D: Position fix that includes only horizontal coordinates (XY).

- Satellites: Number of GPS satellites visible to the receiver and being used to calculate the position.

- HDOP: Horizontal Dilution of Precision.

- VDOP: Vertical Dilution of Precision.

- PDOP: Position Dilution of Precision.

  - Best case value of 1, higher numbers indicate worse PDOP.

  - A value of 2 is good.

  - Higher values (>7) are bad.

  - The best PDOP would occur with one satellite directly overhead and three others evenly spaced about the horizon.

  - PDOP could theoretically be infinite, if all the satellites were in the same plane.

- vAcc: Estimated accuracy of Altitude (i.e. the Z axis).

- Fixed Position: Boolean, indicates if the base station has been configured to a coordinate.

- iTOW: GPS Time of Week.

- rpE: Rover Relative Position to Base Station, Easting.

- rpN: Rover Relative Position to Base Station, Northing.

- EPSG Geodetic Parameter Dataset: Public registry of geodetic datums, spatial reference systems, Earth ellipsoids, coordinate transformations and related units of measurement, originated by a member of the European Petroleum Survey Group (EPSG) in 1985.

- EPSG:4326: WGS 84, latitude/longitude coordinate system based on the Earth's center of mass, used by the Global Positioning System among others.

- WKT: Well-known text markup language for representing spatial reference systems and transformations between spatial reference system.

- WGS83: EPSG:4326

- NAD84:EPSG:4269

# System Requirements

The following outlines the objectives and system requirements for the Ovation Demonstration System.

## Objectives

The goals for the Ovation Demonstration System include the following:

- Upon initiation, such as a button press, automatically perform the following steps:

  - Read GPS location of the ball.

  - Calculate the location using a local grid coordinate system.

    - Cartesian (XYZ) grid based on (0, 0) being a designated corner of the green.

  - Determine the ideal putt trajectory.

  - Display information to a user.

- Use a Windows Laptop

  - Developed in LabView and executed as a Windows x86 compiled executable.

- Use two RoboDot RTK GPS Receivers to form a base station, rover mapping system for highly accurate GPS readings.

- Use a portable Wi-Fi router to establish a Wi-Fi network in field.

## System Context

### Use Cases

The following use-case table identifies the entities that use the system and for what purposes.

| Actor | Use Case | Description |
| --- | --- | --- |
| User | User Places Rover at Putt Location | The User places the Rover at the location of the ball (i.e., where they will be putting from) |
| User | User Requests Putt Instructions | The user will click on a button on the LabView Demo Application to initiate the |
| LabView Demo Application | Display Putt Instructions | The LabView Demo Application will display the putt instructions as outlined below. |
| Base Station | Send GPS Correction (RTK) | The Base Station (RoboDot) sends the GPS Correction (RTK) to the Rover. |
| Rover | Sends Accurate GPS | The Rover will apply the RTK corrections to the generated GPS coordinates and send these accurate GPS coordinates to the LabView Demo Application |
| LabView Demo Application | Send GPS Coordinates Receive Cartesian (XY) Coordinates | The LabView Demo Application, through a special module developed in LabView, will receive the GPS coordinates, convert them to a state plane coordinates, and then convert the state plane coordinates to a local XY grid system used by the advanced 3D simulation system. |

# System Architecture

The follow diagram highlights the functional parts of the system and connection/communication between the different components.

## LabView Demo

### Windows Tablet/Laptop

The demo project will use the existing Ovation laptop for the demo purposes.

The general requirements to operate the LabView software are:

- Processor: Intel Core i5 or greater

- Memory: 16GB minimum

- Hard Drive: 256GB or larger

- Operating System: Windows 10 and above

- Software Requirements

  - LabView Runtime Version 2021 SP1

- 2.4GHz Wi-Fi

- A potential tablet that would meet the requirements is the Microsoft Surface Pro 7+ as shown:

### LabView Application User Interface

The objectives of the application user interface is to display the following information:

Putt Instructions such as

  - Distance to Hole (Direct Line)

  - Distance to Hole (Ideal Putt)

  - Direction (Relative to Hole)

3D Topographical Rendering of the Green

Ideal Putt Trajectory

  - Preferably overlaid on the 3D Topographical Rendering

#### Example Screen Layout

#### Setting the Position of the Hole

This process to the hole position will be used in the following conditions:

- A different putting green from the last operation is being used.

- The hole has been relocated.

It is anticipated that the LabView software will store/recall the location of the hole.

## GPS System

The GPS system will utilize two of the Robota RoboDot RTK GPS Receivers.

https://www.robota.us/solutions/RoboDot

#### Base Station

One of the RoboDots will be configured as follows:

Mode: RoboDot Base Mode

GPS Correction: Sent over the Wi-Fi network as an NTRIP Server

Absolute Position

  - Absolute Position will be determined using “Average Position” over a period of 4 to 8 hours.

    - Average Position will calculate the base position by averaging readings.

  - Absolute Position can be saved in the RoboDot software and recalled later allowing for:

    - Day 1: Configuration (determining Absolute Position through “Average Position” mode) and testing.

    - Day 2: Demonstration, recall saved position and verify accuracy (TBD)

#### Rover

The Rover will be used to determine the location of the ball (i.e. where the putt will originate from).

Mode: Rover (Status page -> Make Rover)

GPS Correction: Received from Base Station over the Wi-Fi network as an NTRIP Client

*Optional: A mechanical model could be designed and 3D printed allowing the Rover to be placed over the ball.

## Wi-Fi Router

A small, portable Wi-Fi router will allow each of the demonstration system devices to connect and communicate between the different components. The requirements for the Wi-Fi router are:

#### Requirements

2.4GHz Wi-Fi

WPA2 Personal Encryption

Range: Maximum

Embedded DHCP Server with support for static IP assignments

#### Static IP Assignments

Each device will be set up in the Wi-Fi router with its MAC address recorded to provide a static IP address (i.e., the same IP address every time the device connects).

Static IP Assignments

| Device | MAC Address | Static IP Address |
| --- | --- | --- |
| Wi-Fi Router | 94:83:C4:2F:DD:04 | 192.168.8.1 |
| Laptop | E8:FB:1C:B6:F9:5B | 192.168.8.2 |
| Base Station (RoboDot #271) | C8:C9:A3:FD:E4:14 | 192.168.8.3 |
| Rover (RoboDot #) | 30:AE:A4:2D:1D:B0 | 192.168.8.4 |

# Test Modes

The LabView Demo Software will support the following test modes allowing for evaluation of functionality of the system and/or debugging issues in-field.

## Connectivity Test

Displays the information for each device.

Wi-Fi Router

Rover

Base Station

The screen will display the following information:

Device Name

IP Address

Connectivity Status

  - Probably accomplished via an ICMP (ping) or an http GET request.

##### Other Specifications

Data updates every 10 seconds.

Constants or set configuration variables are pulled from the LabView environment.

## Rover Status

Displays GPS information from the Rover including:

Device Information

  - Mode

  - Designator

  - Device

  - IP Address

  - Version

  - Shots

GPS Information

  - Latitude

  - Longitude

  - Altitude (meters)

  - Fix Type

  - Satellites

  - HDOP

  - VDOP

  - PDOP

  - vAcc

  - Fixed Position

  - iTOW

  - rpE

  - rpN

Health

  - Run Time

  - Battery

  - Network OK

  - Connected to VRS

  - Connected to AP

  - Memory Card

  - Receive RTCM

State Plane Coordinate System being used.

Translated XY Coordinates in State Plane Coordinate System.

Translated XY Coordinates in local grid system.

Translated Z Height in local grid system.

## Base Station Status

Displays GPS information from the Rover including:

Device Information

  - Mode

  - Designator

  - Device

  - IP Address

  - Version

  - Shots

GPS Information

  - Latitude

  - Longitude

  - Altitude (meters)

  - Fix Type

  - Satellites

  - HDOP

  - VDOP

  - PDOP

  - vAcc

  - Fixed Position

  - iTOW

  - rpE

  - rpN

Health

  - Run Time

  - Battery

  - Network OK

  - Connected to VRS

  - Connected to AP

  - Memory Card

  - Receive RTCM

# Demo Process

The following outlines the major activities in using the Demonstration System.

## Setup

Steps

- Place the Wi-Fi Router and power it on.

- Place the Base Station at a known location that is pre-designated at the course.

Turn on the Base Station.

#### Set Hole Position

Place the RoboDot on the location of the hole.

Click on the Set Hole Position on the LabView Application.

## Generating Putt Instructions

This process will occur when operating the demonstration system in a typical environment, after setup activities have been completed.

Steps

- Place the RoboDot over the ball (using the RoboDot stand) or next to the ball.

- Click the “Go” button on the LabView Application

- LabView Application verifies the following:

  - Base Station fix type is set to Surveyed.

  - Rover is connected to VRS Server.

  - PDOP is less than 2 value.

LabView Application generates putting instructions using the location of the hole and the coordinates as received from the RoboDot Rover.

# Golf Course Information

## Riverside Country Club

https://www.riversidecountryclub.org

#### Address

2701 N University Ave

Provo, UT 84604

#### GPS Coordinates

- Latitude: 40.26885607687802

- Longitude: -111.65855888637971

#### State Plane Coordinate System

- Zone: UT_C

- Zone Name: Utah Central

- FIPS Zone: 4302

- EPSG: 6619

  - https://epsg.io/6619

- GPS Coordinate Format: NAD83

## Shorehaven Golf Club

https://www.shorehavengc.org

#### Address

​14 Canfield Ave

Norwalk CT 06855-2817

#### GPS Coordinates

- Latitude: 41.0932723

- Longitude: -73.3915444

#### State Plane Coordinate System

- Zone: CT

- Zone Name: Connecticut

- EPSG: 26956

  - https://epsg.io/26956

- FIPS Zone: 600

- GPS Coordinate Format: NAD83

# Coordinate Transformations

The following diagram shows the flow of GPS data from both the Ground Based LiDAR system and the RoboDot through coordinate transformation systems to the local grid system. The physics engine utilizes the local grid system to generate putt instructions.

The RoboDot will be generating GPS coordinates in WGS84 or EPSG:4326.

## Coordinate systems

State Plane Coordinate Systems provide information to project Spherical coordinates onto a flat coordinate system.

Extracted from http://www.sharpgis.net/post/Spatial-references2c-coordinate-systems2c-projections2c-datums2c-ellipsoids-e28093-confusing.

A coordinate system is simply put a way of describing a spatial property relative to a center. There is more than one way of doing this:

- The Geocentric coordinate system is based on a normal (X,Y,Z) coordinate system with the origin at the center of Earth. This is the system that GPS uses internally for doing it calculations, but since this is very unpractical to work with as a human being (due to the lack of well-known concepts of east, north, up, down) it is rarely displayed to the user but converted to another coordinate system.

- The Spherical or Geographic coordinate system is probably the most well-known. It is based on angles relative to a prime meridian and Equator usually as Longitude and Latitude. Heights are usually given relative to either the mean sea level or the datum.

- The Cartesian coordinate system is defined as a “flat” coordinate system placed on the surface of Earth.  In some projections it’s not flat in the sense that it follows the earth’s curvature in one direction and has a known scale-error in the other direction relative to the distance of the origin. The most well-known coordinate system is the Universal Transverse Mercator (UTM), but surveyors define their own little local flat coordinate systems all the time. It is very easy to work with, fairly accurate over small distances making measurements such as length, angle and area very straightforward. Cartesian coordinate systems are strongly connected to projections.

Datums and ellipsoids

Some of the common properties of the above coordinate systems are that they are all relative to the center of Earth and except the Geocentric coordinate system, uses a height system relative to the surface of the earth.

This poses two immediate problems:

- Where is the center of the earth

- What is the shape of the earth?

If we define the surface of Earth as being at the mean sea level (often referred to as the Geoid), we don’t get a spheroid or even an ellipsoid. Because of gravitational changes often caused by large masses such as mountain ranges etc, Earth is actually very irregular with variations of +/- 100 meters. Since this is not very practical to work with as a model of earth, we usually use an ellipsoid for approximation. The ellipsoid is defined by its semi-major axis, and either the flattening of the ellipoid or the semi-minor axis.

The center and orientation of the ellipsoid is what we call the datum. So the datum defines an ellipsoid and through the use of a set of points on the ground that we relate to points on the ellipsoid, we define the center of the Earth.  This poses another problem, because continental drift moves the points used to define the points around all the time. This is why the name of a datum usually have a year in it, often referring to the position of those points January 1st of that year (although that may vary).

There are a vast amount of datums, some used for measurements all over the world, and other local datums defined so they fit very well with a local area. Some common ones are: World Geodetic Datum 1984 (WGS84), European Datum 1950 (ED50) and North American Datum 1983 (NAD83).

The most well-known is WGS84 used by the GPS systems today.

Projections

The earth isn’t flat, and there is no simple way of putting it down on a flat paper map (or these days onto a computer screen), so people have come up with all sorts of ingenious solutions each with their pros and cons.

Spatial reference

The spatial reference is a combination of all the above. It defines an ellipsoid, a datum using that ellipsoid, and either a geocentric, geographic or projection coordinate system. The projection also always has a geographic coordinate system associated with it. The European Petroleum Survey Group (EPSG) has a huge set of predefined spatial references, each given a unique ID. These IDs are used throughout the industry.

WGS84: The spatial reference ID of this is EPSG:4326.

Spatial References are often defined in a Well-known format defining all these parameters.

### Additional Reference Information

http://www.sharpgis.net/post/Spatial-references2c-coordinate-systems2c-projections2c-datums2c-ellipsoids-e28093-confusing

Another article explaining GPS georeferencing (spheroid, datum, projecting).

https://www.directionsmag.com/article/4048

Geodetic Distance vs Grid Distance

https://www.geoweeknews.com/blogs/vol13no16-state-plane-coordinates

## Tools

https://geodesy.noaa.gov/NCAT/

- NGS Coordinate Conversion and Transformation Tool: The industry standard for North America coordinate conversion and transformation. Based on NAD84.

https://proj.org/en/9.2/

- This is probably the tool we are going to want to use. There is a python interface built for it as well.

https://proj.org/en/9.2/operations/projections/index.html

- Projections

https://proj.org/en/9.2/usage/ellipsoids.html#ellipsoids

- Ellipsoids

https://pypi.org/project/pyproj/

- Python interface to PROJ

https://hub.arcgis.com/datasets/esri::usa-state-plane-zones-nad83/explore

https://products.aspose.app/gis/transformation